COIMBATORE INSTITUTE OF TECHNOLOGY

CIVIL AERODOME POST, AVINASHI ROAD, COIMBATORE-641014

NEXERA 2K26

ROBOVERSE

CYBER KICK

OBJECTIVE

Cyber Kick is a competitive robotics football challenge where teams design, build, and control a robot capable of fast maneuvers, ball handling, and strategic movement. The event tests system integration, dynamic control, wireless communication reliability, and tactical gameplay in real time

ARENA

The game field consists of a soccer arena with the following specifications:

- The arena dimensions are 180 cm × 120 cm (length × breadth).

- The field is marked with white lines indicating boundaries and the center line.

- Two goals of dimensions 60 cm × 40 cm are positioned at opposite ends of the field.

- The penalty area is marked 40 cm from each goal line.

- The center circle has a radius of 30 cm from the midpoint of the field.

- The playing surface is green artificial turf or similar material.

- White boundary markers prevent the ball from rolling out.

Note: The dimensions of the arena will be accurate to within 5% or 20 mm, whichever is less.

GAMEPLAY

The gameplay follows standard soccer rules adapted for autonomous robots:

- Match Duration: Each match consists of two halves of  180seconds    each, with a  30‑second halftime break is provided for minor adjustments.

- Starting Position:Robots must start from their designated starting zone..

- Ball Possession: The robot that successfully tracks and contacts the ball gains possession.

- Scoring: A goal is scored when the ball completely crosses the goal line and enters the goal. The robot must navigate and shoot the ball into the opponent's goal.

- Out of Play: If the ball exits the field boundaries, play is resumed and the referee will reset their position.

- Penalty Reset: If a robot fouls or goes out of bounds, the opposing robot receives possession at the center line.

- Match Completion: The match ends after both halves are played. The team with the highest number of goals wins.

Total match time:6minutes (180 seconds per half + 30seconds halftime break).

FIELD MARKINGS

- Goal Line: End boundaries where goals are positioned (120 cm apart)

- Penalty Area: Zone 40 cm from goal line where penalties apply

- Center Line: Divides the field into two halves (90 cm from each goal)

- Center Circle: 30 cm radius circle at the field center for kickoff positioning

- Boundary Lines: White markers indicating out-of-play zones

ROBOT SPECIFICATIONS

- The robot must fit within a 30 cm × 30 cm × 30 cm box.

- The maximum robot weight allowed is 4 kg.

- The robot must be self-powered using an on-board battery only (maximum supply voltage 12V DC).

- The robot may be wired or wireless but must be manually controlled.

- Only one operator per robot is allowed during a match. Operators may only be changed between rounds.

- Robots are allowed to push or hit the ball only.They cannot pick, scoop, lift, or grab the ball.

- Any mechanism designed solely for damaging other robots is strictly prohibited.

- Human intervention during match is restricted.

GAME RULES

- Calibration Time: 2 minutes before each match for sensor calibration. No code changes allowed after calibration begins.

- One Robot Per Team: Only one robot per team is allowed on the field.

- No Manual Intervention: Team members cannot touch the robot or enter the field during active play.

- Referee Authority: Only one team member may stand near the field boundary at the start. Organizers' decisions are final.

RESTARTS

- Maximum Restarts: Up to 3 restarts are allowed per match per team.

- Restart Position: Upon restart, the robot is repositioned at the center line without penalty.

- Timer Continuation: The match timer does not reset or pause during restarts.

- Hardware Adjustments: Teams may adjust sensors or hardware configurations during restart. Feeding arena information is forbidden.

- No Performance Enhancement: Significant modifications to robot weight or structure during restarts are not allowed.

GENERAL RULES

- Each team may have a maximum of 4 members, but only one robot can be deployed during a match.

- Each match consists of two halves of 180 seconds each. There will be a 30 seconds  half-time break for minor maintenance and adjustment.

- Arena dimensions and ball specifications will be shared on the event day.

- Robots must begin from their designated starting zone. The timer starts and stops only when signaled by the referee.

- Robots may only push or hit the ball to move it toward the goal.

- A goal is awarded only when the ball fully crosses the goal line. If a robot or the ball goes out of the arena, the referee will reset their position.

- Excessive ramming, intentional flipping, or any attempt to damage an opponent’s robot will lead to penalties or disqualification.

- The decisions made by the organizing committee and the referee are final.

TEAM SPECIFICATION

- Maximum 4 participants per team; cross-institute teams are allowed.

- All team members must carry valid student ID cards at the venue.

- Team Lead must be designated for communication with organizers.

ELIGIBILITY

- All students possessing a valid student ID card are eligible to participate.

- There are no restrictions on year of study or department (for cross-institute teams).

CERTIFICATE

- Certificates will be provided to all participants on the day of the event.

- Winners will receive special recognition and prizes.

THANK YOU

We wish all teams the best of luck! Looking forward to witnessing exciting autonomous soccer matches!

For queries contact: NEXERA 2K26 Organizing Committee, Coimbatore Institute of Technology